Spartan-7 FPGAs

|  |  |  |  |  | I/O Optimization at the Lowest Cost and Highest Performance-per-Watt |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | (1.0V, 0.95V) |  |  |  |  |  |
|  |  |  |  | Part Number | XC7S6 | XC7S15 | XC7S25 | XC7S50 | XC7S75 | XC7S100 |
|  |  |  |  | Logic Cells | 6,000 | 12,800 | 23,360 | 52,160 | 76,800 | 102,400 |
|  | Logic Resources |  |  | Slices | 938 | 2,000 | 3,650 | 8,150 | 12,000 | 16,000 |
|  |  |  |  | CLB Flip-Flops | 7,500 | 16,000 | 29,200 | 65,200 | 96,000 | 128,000 |
|  |  |  | Max. Distributed RAM (Kb) |  | 70 | 150 | 313 | 600 | 832 | 1,100 |
|  | Memory Resources | Block RAM/FIFO w/ ECC (36 Kb each) |  |  | 5 | 10 | 45 | 75 | 90 | 120 |
|  |  |  | Total Block RAM (Kb) |  | 180 | 360 | 1,620 | 2,700 | 3,240 | 4,320 |
|  | Clock Resources | Clock Mgmt Tiles (1 MMCM + 1 PLL) |  |  | 2 | 2 | 3 | 5 | 8 | 8 |
|  | I/O Resources |  | Max. Single-Ended I/O Pins |  | 100 | 100 | 150 | 250 | 400 | 400 |
|  |  |  | Max. Differential I/O Pairs |  | 48 | 48 | 72 | 120 | 192 | 192 |
|  | Embedded Hard IP |  |  | DSP Slices | 10 | 20 | 80 | 120 | 140 | 160 |
|  |  | Analog Mixed Signal (AMS) / XADC |  |  | 0 | 0 | 1 | 1 | 1 | 1 |
|  | Resources |  |  |  |  |  |  |  |  |  |
|  |  | Configuration AES / HMAC Blocks |  |  | 0 | 0 | 1 | 1 | 1 | 1 |
|  |  |  | Commercial Temp (C) |  | -1,-2 | -1,-2 | -1,-2 | -1,-2 | -1,-2 | -1,-2 |
|  | Speed Grades |  | Industrial Temp (I) |  | -1,-2,-1L | -1,-2,-1L | -1,-2,-1L | -1,-2,-1L | -1,-2,-1L | -1,-2,-1L |
|  |  |  | Expanded Temp (Q) |  | -1 | -1 | -1 | -1 | -1 | -1 |
|  |  |  | Body Area | Ball Pitch |  |  |  |  |  |  |
|  |  | Package(1) | (mm) | (mm) |  |  | Available User I/O: 3.3V SelectIO™ HR I/O |  |  |  |
|  |  | CPGA196 | 8x8 | 0.5 | 100 | 100 |  |  |  |  |
|  |  | CSGA225 | 13x13 | 0.8 | 100 | 100 | 150 |  |  |  |
|  |  | CSGA324 | 15x15 | 0.8 |  |  | 150 | 210 |  |  |
|  |  | FTGB196 | 15x15 | 1.0 | 100 | 100 | 100 | 100 |  |  |
|  |  | FGGA484 | 23x23 | 1.0 |  |  |  | 250 | 338 | 338 |
|  |  | FGGA676 | 27x27 | 1.0 |  |  |  |  | 400 | 400 |

Notes:

- Packages with the same last letter and number sequence, e.g., A484, are footprint compatible with all other Spartan-7 devices with the same sequence. The footprint compatible devices within this family are outlined.

Page 2

Artix-7 FPGAs

|  |  |  |  |  |  |  | Transceiver Optimization at the Lowest Cost and Highest DSP Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | (1.0V, 0.95V, 0.9V) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Part Number |  | XC7A12T |  | XC7A15T |  |  |  | XC7A25T |  | XC7A35T |  |  |  | XC7A50T |  | XC7A75T |  |  |  | XC7A100T |  | XC7A200T |  |  |  |
| Logic |  |  |  |  |  | Logic Cells | 12,800 |  | 16,640 |  |  |  | 23,360 |  | 33,280 |  |  |  | 52,160 |  | 75,520 |  |  |  | 101,440 |  | 215,360 |  |  |  |
|  |  |  |  |  |  | Slices | 2,000 |  | 2,600 |  |  |  | 3,650 |  | 5,200 |  |  |  | 8,150 |  | 11,800 |  |  |  | 15,850 |  | 33,650 |  |  |  |
| Resources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | CLB Flip-Flops |  | 16,000 |  | 20,800 |  |  |  | 29,200 |  | 41,600 |  |  |  | 65,200 |  | 94,400 |  |  |  | 126,800 |  | 269,200 |  |  |  |
| Memory |  | Maximum Distributed RAM (Kb) |  |  |  |  | 171 |  | 200 |  |  |  | 313 |  | 400 |  |  |  | 600 |  | 892 |  |  |  | 1,188 |  | 2,888 |  |  |  |
|  |  | Block RAM/FIFO w/ ECC (36 Kb each) |  |  |  |  | 20 |  | 25 |  |  |  | 45 |  | 50 |  |  |  | 75 |  | 105 |  |  |  | 135 |  | 365 |  |  |  |
| Resources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Total Block RAM (Kb) |  | 720 |  | 900 |  |  |  | 1,620 |  | 1,800 |  |  |  | 2,700 |  | 3,780 |  |  |  | 4,860 |  | 13,140 |  |  |  |
| Clock Resources |  |  |  | CMTs (1 MMCM + 1 PLL) |  |  | 3 |  | 5 |  |  |  | 3 |  | 5 |  |  |  | 5 |  | 6 |  |  |  | 6 |  | 10 |  |  |  |
| I/O Resources |  | Maximum Single-Ended I/O |  |  |  |  | 150 |  | 250 |  |  |  | 150 |  | 250 |  |  |  | 250 |  | 300 |  |  |  | 300 |  | 500 |  |  |  |
|  |  | Maximum Differential I/O Pairs |  |  |  |  | 72 |  | 120 |  |  |  | 72 |  | 120 |  |  |  | 120 |  | 144 |  |  |  | 144 |  | 240 |  |  |  |
|  |  |  |  |  |  | DSP Slices | 40 |  | 45 |  |  |  | 80 |  | 90 |  |  |  | 120 |  | 180 |  |  |  | 240 |  | 740 |  |  |  |
|  |  |  |  |  | PCIe® Gen2(1) |  | 1 |  | 1 |  |  |  | 1 |  | 1 |  |  |  | 1 |  | 1 |  |  |  | 1 |  | 1 |  |  |  |
| Embedded |  | Analog Mixed Signal (AMS) / XADC |  |  |  |  | 1 |  | 1 |  | 1 |  |  |  | 1 |  | 1 |  |  |  | 1 |  | 1 |  |  |  | 1 |  |  |  |
| Hard IP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Configuration AES / HMAC Blocks |  |  |  |  | 1 |  | 1 |  |  |  | 1 |  | 1 |  |  |  | 1 |  | 1 |  |  |  | 1 |  | 1 |  |  |  |
| Resources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | GTP Transceivers (6.6 Gb/s Max |  |  |  | 2 |  | 4 |  |  |  | 4 |  | 4 |  |  |  | 4 |  | 8 |  |  |  | 8 |  | 16 |  |  |  |
|  |  |  |  |  |  | Rate)(2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Commercial Temp (C) |  |  | -1, -2 |  | -1, -2 |  |  |  | -1, -2 |  | -1, -2 |  |  |  | -1, -2 |  | -1, -2 |  |  |  | -1, -2 |  | -1, -2 |  |  |  |
| Speed Grades |  |  |  |  | Extended Temp (E) |  | -2L, -3 |  | -2L, -3 |  |  |  | -2L, -3 |  | -2L, -3 |  |  |  | -2L, -3 |  | -2L, -3 |  |  |  | -2L, -3 |  | -2L, -3 |  |  |  |
|  |  |  |  |  | Industrial Temp (I) |  | -1, -2, -1L |  | -1, -2, -1L |  |  |  | -1, -2, -1L |  | -1, -2, -1L |  |  |  | -1, -2, -1L |  | -1, -2, -1L |  |  |  | -1, -2, -1L |  | -1, -2, -1L |  |  |  |
|  |  |  | Package(3), (4) |  | Dimensions | Ball Pitch |  |  |  |  |  |  | Available User I/O: 3.3V SelectIO™ HR I/O (GTP Transceivers) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | (mm) | (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CPG236 |  | 10 x 10 | 0.5 |  |  | 106 (2) |  |  |  |  |  | 106 (2) |  |  |  | 106 (2) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CPG238 |  | 10 x 10 | 0.5 | 112 (2) |  |  |  |  |  | 112 (2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CSG324 |  | 15 x 15 | 0.8 |  |  | 210 (0) |  |  |  |  |  | 210 (0) |  |  |  | 210 (0) |  | 210 (0) |  |  |  | 210 (0) |  |  |  |  |  |
|  |  |  | CSG325 |  | 15 x 15 | 0.8 | 150 (2) |  | 150 (4) |  |  |  | 150 (4) |  | 150 (4) |  |  |  | 150 (4) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | FTG256 |  | 17 x 17 | 1.0 |  |  | 170 (0) |  |  |  |  |  | 170 (0) |  |  |  | 170 (0) |  | 170 (0) |  |  |  | 170 (0) |  |  |  |  |  |
|  |  |  | SBG484 |  | 19 x 19 | 0.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 285 (4) |  |  |  |
| Footprint |  | FGG484(5) |  |  | 23 x 23 | 1.0 |  |  | 250 (4) |  |  |  |  |  | 250 (4) |  |  |  | 250 (4) |  | 285 (4) |  |  |  | 285 (4) |  |  |  |  |  |
| Compatible |  | FBG484(5) |  |  | 23 x 23 | 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 285 (4) |  |  |  |
| Footprint |  | FGG676(6) |  |  | 27 x 27 | 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 300 (8) |  |  |  | 300 (8) |  |  |  |  |  |
| Compatible |  | FBG676(6) |  |  | 27 x 27 | 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 400 (8) |  |  |  |
|  |  |  | FFG1156 |  | 35 x 35 | 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 500 (16) |  |  |  |
|  |  |  | Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | 1. | Supports PCI Express Base 2.1 specification at Gen1 and Gen2 data rates. |
| --- | --- | --- |
|  | 2. | Represents the maximum number of transceivers available. Note that the majority of devices are available without transceivers. See the Package section of this table for details. |
|  | 3. | Leaded package option available for all packages. See DS180, 7 Series FPGAs Overview for package details. |
|  | 4. | Device migration is available within the Artix-7 family for like packages but is not supported between other 7 series families. |
| Page 3 | 5. | Devices in FGG484 and FBG484 are footprint compatible. |
|  | 6. | Devices in FGG676 and FBG676 are footprint compatible. |

Kintex-7 FPGAs

Optimized for Best Price-Performance (1.0V, 0.95V, 0.9V)

|  |  |  | Part Number | XC7K70T | XC7K160T | XC7K325T | XC7K355T | XC7K410T | XC7K420T | XC7K480T |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EasyPath™ Cost Reduction Solutions(1) |  |  | — | — | XCE7K325T | XCE7K355T | XCE7K410T | XCE7K420T | XCE7K480T |
|  |  |  | Slices | 10,250 | 25,350 | 50,950 | 55,650 | 63,550 | 65,150 | 74,650 |
| Logic Resources |  |  | Logic Cells | 65,600 | 162,240 | 326,080 | 356,160 | 406,720 | 416,960 | 477,760 |
|  |  |  | CLB Flip-Flops | 82,000 | 202,800 | 407,600 | 445,200 | 508,400 | 521,200 | 597,200 |
| Memory | Maximum Distributed RAM (Kb) |  |  | 838 | 2,188 | 4,000 | 5,088 | 5,663 | 5,938 | 6,788 |
|  | Block RAM/FIFO w/ ECC (36 Kb each) |  |  | 135 | 325 | 445 | 715 | 795 | 835 | 955 |
| Resources |  |  |  |  |  |  |  |  |  |  |
|  |  | Total Block RAM (Kb) |  | 4,860 | 11,700 | 16,020 | 25,740 | 28,620 | 30,060 | 34,380 |
| Clock Resources |  | CMTs (1 MMCM + 1 PLL) |  | 6 | 8 | 10 | 6 | 10 | 8 | 8 |
| I/O Resources |  | Maximum Single-Ended I/O |  | 300 | 400 | 500 | 300 | 500 | 400 | 400 |
|  | Maximum Differential I/O Pairs |  |  | 144 | 192 | 240 | 144 | 240 | 192 | 192 |
|  |  |  | DSP48 Slices | 240 | 600 | 840 | 1,440 | 1,540 | 1,680 | 1,920 |
| Integrated IP |  |  | PCIe® Gen2(2) | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Analog Mixed Signal (AMS) / XADC |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Resources |  |  |  |  |  |  |  |  |  |  |
|  | Configuration AES / HMAC Blocks |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | GTX Transceivers (12.5 Gb/s Max Rate) |  |  | 8 | 8 | 16 | 24 | 16 | 32 | 32 |
|  |  | Commercial Temp (C) |  | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 |
| Speed Grades |  | Extended Temp (E) |  | -2L, -3 | -2L, -3 | -2L, -3 | -2L, -3 | -2L, -3 | -2L, -3 | -2L, -3 |
|  |  | Industrial Temp (I) |  | -1, -2 | -1, -2, -2L | -1, -2, -2L | -1, -2, -2L | -1, -2, -2L | -1, -2, -2L | -1, -2, -2L |
|  | Package(3) | Dimensions | Ball Pitch |  |  | Available User I/O: 3.3V HR I/O, 1.8V HP I/Os (GTX) |  |  |  |  |
|  |  | (mm) | (mm) |  |  |  |  |  |  |  |
|  | FBG484(4) | 23 x 23 | 1.0 | 185, 100 (4) | 185, 100 (4) |  |  |  |  |  |
| Footprint | FBG676(4) | 27 x 27 | 1.0 | 200, 100 (8) | 250, 150 (8) | 250, 150 (8) |  | 250, 150 (8) |  |  |
| Compatible | FFG676 | 27 x 27 | 1.0 |  | 250, 150 (8) | 250, 150 (8) |  | 250, 150 (8) |  |  |
| Footprint | FBG900(4) | 31 x 31 | 1.0 |  |  | 350, 150 (16) |  | 350, 150 (16) |  |  |
| Compatible | FFG900 | 31 x 31 | 1.0 |  |  | 350, 150 (16) |  | 350, 150 (16) |  |  |
|  | FFG901 | 31 x 31 | 1.0 |  |  |  | 300, 0 (24) |  | 380, 0 (28) | 380, 0 (28) |
|  | FFG1156 | 35 x 35 | 1.0 |  |  |  |  |  | 400, 0 (32) | 400, 0 (32) |

Notes:

- EasyPath™ solutions provide a fast and conversion-free path for cost reduction.

- Hard block supports PCI Express Base 2.1 specification at Gen1 and Gen2 data rates. Gen3 supported with soft IP.

- See DS180, 7 Series FPGAs Overview, for package details.

- GTX transceivers in FB packages support the following maximum data rates: 10.3Gb/s in FBG484; 6.6Gb/s in FBG676 and FBG900. See DS182, Kintex-7 FPGAs Data Sheet: DC and AC Switching Characteristics, for details.

Page 4

Virtex-7 FPGAs

|  |  |  |  |  | Optimized for Highest System Performance and Capacity |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | (1.0V) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Part Number | XC7V585T | XC7V2000T | XC7VX330T | XC7VX415T | XC7VX485T | XC7VX550T | XC7VX690T | XC7VX980T | XC7VX1140T | XC7VH580T | XC7VH870T |
|  | EasyPath™ Cost Reduction Solutions(1) |  |  |  | XCE7V585T | — | XCE7VX330T | XCE7VX415T | XCE7VX485T | XCE7VX550T | XCE7VX690T | XCE7VX980T | — | — | — |
| Logic |  |  |  | Slices | 91,050 | 305,400 | 51,000 | 64,400 | 75,900 | 86,600 | 108,300 | 153,000 | 178,000 | 90,700 | 136,900 |
|  |  |  |  | Logic Cells | 582,720 | 1,954,560 | 326,400 | 412,160 | 485,760 | 554,240 | 693,120 | 979,200 | 1,139,200 | 580,480 | 876,160 |
| Resources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CLB Flip-Flops |  | 728,400 | 2,443,200 | 408,000 | 515,200 | 607,200 | 692,800 | 866,400 | 1,224,000 | 1,424,000 | 725,600 | 1,095,200 |
| Memory | Maximum Distributed RAM (Kb) |  |  |  | 6,938 | 21,550 | 4,388 | 6,525 | 8,175 | 8,725 | 10,888 | 13,838 | 17,700 | 8,850 | 13,275 |
|  | Block RAM/FIFO w/ ECC (36 Kb each) |  |  |  | 795 | 1,292 | 750 | 880 | 1,030 | 1,180 | 1,470 | 1,500 | 1,880 | 940 | 1,410 |
| Resources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Total Block RAM (Kb) |  |  | 28,620 | 46,512 | 27,000 | 31,680 | 37,080 | 42,480 | 52,920 | 54,000 | 67,680 | 33,840 | 50,760 |
| Clocking |  | CMTs (1 MMCM + 1 PLL) |  |  | 18 | 24 | 14 | 12 | 14 | 20 | 20 | 18 | 24 | 12 | 18 |
| I/O |  | Maximum Single-Ended I/O |  |  | 850 | 1,200 | 700 | 600 | 700 | 600 | 1,000 | 900 | 1,100 | 600 | 300 |
| Resources | Maximum Differential I/O Pairs |  |  |  | 408 | 576 | 336 | 288 | 336 | 288 | 480 | 432 | 528 | 288 | 144 |
|  |  |  |  | DSP Slices | 1,260 | 2,160 | 1,120 | 2,160 | 2,800 | 2,880 | 3,600 | 3,600 | 3,360 | 1,680 | 2,520 |
|  |  |  |  | PCIe® Gen2(2) | 3 | 4 | — | — | 4 | — | — | — | — | — | — |
|  |  |  |  | PCIe Gen3 | — | — | 2 | 2 | — | 2 | 3 | 3 | 4 | 2 | 3 |
| Integrated IP | Analog Mixed Signal (AMS) / XADC |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Resources | Configuration AES / HMAC Blocks |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | GTX Transceivers (12.5 Gb/s Max Rate)(3) |  |  |  | 36 | 36 | — | — | 56 | — | — | — | — | — | — |
|  | GTH Transceivers (13.1 Gb/s Max Rate)(4) |  |  |  | — | — | 28 | 48 | — | 80 | 80 | 72 | 96 | 48 | 72 |
|  | GTZ Transceivers ( 28.05 Gb/s Max Rate) |  |  |  | — | — | — | — | — | — | — | — | — | 8 | 16 |
| Speed |  | Commercial Temp (C) |  |  | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 |
|  |  | Extended Temp (E)(5) |  |  | -2L, -3 | -2L, -2G | -2L, -3 | -2L, -3 | -2L, -3 | -2L, -3 | -2L, -3 | -2L | -2L, -2G | -2L, -2G | -2L, -2G |
| Grades |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Industrial Temp (I) |  |  | -1, -2 | -1 | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1, -2 | -1 | -1 | — | — |
|  | Package(6) | Dimensions |  | Ball Pitch |  |  | Available User I/O: 3.3V HR I/O, 1.8V HP I/Os (GTX, GTH) |  |  |  |  |  |  | 1.8V HP I/O (GTH, GTZ) |  |
|  |  | (mm) |  | (mm) |  |  |  |  |  |  |  |  |  |  |  |
|  | FFG1157(7) | 35 x 35 | 1.0 |  | 0, 600 (20, 0) |  | 0, 600 (0, 20) | 0, 600 (0, 20) | 0, 600 (20, 0) |  | 0, 600 (0, 20) |  |  |  |  |
| Footprint | FFG1761(7) | 42.5 x 42.5 | 1.0 |  | 100, 750 (36, 0) |  | 50, 650 (0, 28) |  | 0, 700 (28, 0) |  | 0, 850 (0, 36) |  |  |  |  |
| Compatible | FHG1761 | 45 x 45 | 1.0 |  |  | 0, 850 (36, 0) |  |  |  |  |  |  |  |  |  |
|  | FLG1925 | 45 x 45 |  | 1.0 |  | 0, 1200 (16, 0) |  |  |  |  |  |  |  |  |  |
|  | FFG1158(7) | 35 x 35 |  | 1.0 |  |  |  | 0, 350 (0, 48) | 0, 350 (48, 0) | 0, 350 (0, 48) | 0, 350 (0, 48) |  |  |  |  |
| Footprint | FFG1926 | 45 x 45 | 1.0 |  |  |  |  |  |  |  | 0, 720 (0, 64) | 0, 720 (0, 64) |  |  |  |
| Compatible | FLG1926 | 45 x 45 | 1.0 |  |  |  |  |  |  |  |  |  | 0, 720 (0, 64) |  |  |
|  | FFG1927(7) | 45 x 45 |  | 1.0 |  |  |  | 0, 600 (0, 48) | 0, 600 (56, 0) | 0, 600 (0, 80) | 0, 600 (0, 80) |  |  |  |  |
| Footprint | FFG1928 | 45 x 45 |  | 1.0 |  |  |  |  |  |  |  | 0, 480 (0, 72) |  |  |  |
| Compatible | FLG1928 | 45 x 45 |  | 1.0 |  |  |  |  |  |  |  |  | 0, 480 (0, 96) |  |  |
| Footprint | FFG1930 | 45 x 45 |  | 1.0 |  |  |  |  | 0, 700 (24, 0) |  | 0, 1000 (0, 24) 0, 900 (0, 24) |  |  |  |  |
| Compatible | FLG1930 | 45 x 45 | 1.0 |  |  |  |  |  |  |  |  |  | 0, 1100 (0, 24) |  |  |
|  | FLG1155 | 35 x 35 |  | 1.0 |  |  |  |  |  |  |  |  |  | 400 (24, 8) |  |
|  | FLG1931 | 45 x 45 |  | 1.0 |  |  |  |  |  |  |  |  |  | 600 (48, 8) |  |
|  | FLG1932 | 45 x 45 | 1.0 |  |  |  |  |  |  |  |  |  |  |  | 300 (72, 16) |

Notes:

- EasyPath™ solutions provide a fast and conversion-free path for cost reduction.

- Hard block supports PCI Express Base 2.1 specification at Gen1 and Gen2 data rates. Gen3 supported with soft IP.

- 12.5 Gb/s support in "-3E", "-2GE" speed/temperature grade; 10.3125 Gb/s support in "2C", "-2LE", and "-2I" speed grade.

- 13.1 Gb/s support in "-3E". "-2GE" speed grade; 11.3 Gb/s support in "2C" , "-2LE" and "-2I" speed/temperature grades.

Page 5

- -2G only applies to Stacked Silicon Interconnect devices and supports 12.5G GTX, 13.1G GTH, 28.05G GTZ with -2 fabric.

- Leaded package options ("FFxxxx"/"FLxxxx"/"FHxxxx") available for all packages. "HCxxxx" is not offered in a leaded option.

- See DS180, 7 Series FPGAs Overview for package details.

Device Ordering Information

- XC

- Xilinx

- Commercial

| XC | 7 | S | ### |  | -1 | FG | G | A | 484 | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Commercial Generation |  | Family | Logic Cells | Speed Grade |  | Package Type | G: RoHS 6/6 | Package | Package | Temperature |
| Xilinx |  |  | In 1K units | -1 | = Slowest | CP: Wire-bond (.5 mm) |  | Designator | Pin Count | Grade |
|  |  |  |  | -L1 = Low Power CS: Wire-bond (.8 mm) |  |  |  |  |  | (C, I, Q) |
|  |  |  |  | -2 | = Mid | FG: Wire-bond (1 mm) |  |  |  |  |
|  |  |  |  |  |  | FT: Wire-bond (1 mm) |  |  |  |  |

| 7 | A | ### |  | -1 | FB | G |
| --- | --- | --- | --- | --- | --- | --- |
| Generation | Family | Logic Cells | Speed Grade |  | Package Type | V: RoHS 6/6 |
|  |  | in 1K Units | -1 | = Slowest | CP: Wire-bond (.5 mm) | G: RoHS 6/6 |
|  |  |  | -L1 = Low Power |  | CS: Wire-bond (.8 mm) | w/Exemption 15 |
|  |  |  | -L2 = Low Power |  | FB: Bare-Die Flip-Chip (1 mm) |  |
|  |  |  | -2 | = Mid | FF: Flip-Chip (1 mm) |  |
|  |  |  | -3 | = Highest | FG: Wire-bond (1 mm) |  |
|  |  |  |  |  | FT: Wire-bond (1 mm) |  |
|  |  |  |  |  | SB: Bare-Die Flip-Chip (.8 mm) |  |

| 484 | C |
| --- | --- |
| Nominal | Temperature |
| Package | Grade |
| Pin Count | (C, E, I) |

| XC | 7 | K | ### |  | -1 | FF | G | 900 | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Xilinx | Generation | Family | Logic Cells | Speed Grade |  | Package Type | V: RoHS 6/6 | Nominal | Temperature |
| Commercial |  |  | in 1K Units | -1 | = Slowest | FB: Bare-Die Flip-Chip (1 mm) | G: RoHS 6/6 | Package | Grade |
|  |  |  |  | -L2 = Low Power |  | FF: Flip-Chip (1 mm) | w/Exemption 15 | Pin Count | (C, E, I) |
|  |  |  |  | -2 | = Mid |  |  |  |  |
|  |  |  |  | -3 | = Highest |  |  |  |  |

|  | XC | 7 | V | ### |  | -1 |  | FF | G | 1156 | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Xilinx | Generation | Family | Logic Cells | Speed Grade |  |  | Package Type | V: RoHS 6/6 | Nominal | Temperature |
| Notes: | Commercial |  |  | in 1K Units | -1 | = Slowest |  | FF: Flip-Chip (1 mm) | G: RoHS 6/6 | Package | Grade |
|  |  |  |  |  | -2 | = Mid |  | FH: Flip-Chip (1 mm) | w/Exemption 15 | Pin Count | (C, E, I) |
| -L1 is the ordering code for the lower power, -1L speed grade. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -L2 = Low Power |  |  | FL: Flip-Chip (1 mm) |  |  |  |
| -L2 is the ordering code for the lower power, -2L speed grade. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -3 | = Highest | HC: Ceramic Flip-Chip (1 mm) |  |  |  |  |
| C = Commercial (Tj = 0°C to +85°C) E = Extended (Tj = 0°C to +100°C) |  |  |  | I = Industrial (Tj = –40°C to +100°C) |  |  |  | Q = Expanded (Tj = –40°C to +125°C) |  |  |  |
|  | Page 6 |  |  |  |  |  |  |  |  |  |  |

Digital Signal Processing Metrics

|  | DSP Slice Count |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Spartan®-7 FPGAs |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XC7S6 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7S15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Speed grade |  |  | -1 | -2 |  |  |  |
| XC7S25 | 80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMAX [MHz] |  |  | 464 | 550 |  |  |  |
| XC7S50 | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Max GMAC/s |  |  | 148 | 176 |  |  |  |
| XC7S75 | 140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7S100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 160 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Artix®-7 FPGAs |  |  |  |  |  |  |  |
| XC7A12T | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Speed grade |  |  | -1 | -2 |  |  | -3 |
| XC7A15T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMAX [MHz] |  |  | 464 | 550 |  |  | 628 |
| XC7A25T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A35T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Max GMAC/s |  |  | 686 | 814 |  |  | 929 |
|  |  | 90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A50T | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A75T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 180 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Kintex®-7 FPGAs |  |  |  |  |  |  |  |
| XC7A100T |  | 240 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A200T |  |  |  |  |  | 740 |  |  |  |  |  |  |  |  |  |  | Speed grade |  |  | -1 | -2 |  |  | -3 |
| XC7K70T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMAX [MHz] |  |  | 464 | 550 |  |  | 741 |
|  |  | 240 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Max GMAC/s |  |  | 1,781 | 2,112 |  |  | 2,845 |
| XC7K160T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 600 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K325T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K355T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1440 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Virtex®-7 FPGAs |  |  |  |  |  |  |  |
| XC7K410T |  | 1540 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Speed grade |  |  | -1 | -2 |  |  | -3 |
| XC7K420T |  | 1680 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FMAX [MHz] |  |  | 547 | 650 |  |  | 741 |
| XC7K480T |  | 1920 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7V585T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Max GMAC/s |  |  | 2,756 | 3,276 |  |  | 3,734 |
|  |  | 1260 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7V2000T |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2160 |  |  |  |  |  |  |  |  |  |
| XC7VX330T |  | 1120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX415T |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2160 |  |  |  |  |  |  |  |  |  |
| XC7VX485T |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2800 |  |  |  |  |  |  |  |  |  |
| XC7VX550T |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5760 |  |  |  |  |  |  |  |  |  |
| XC7VX690T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3600 |  |  |  |
| XC7VX980T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3600 |  |  |  |
| XC7VX1140T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3360 |  |  |  |
| XC7VH580T |  | 1680 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VH870T |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2520 |  |  |  |  |  |  |  |  |  |

For more information, refer to: UG479, 7 Series FPGAs DSP48E1 Slice User Guide

Important: Verify all data in this document with the device data sheets found at www.xilinx.com

Page 7

Block RAM Metrics

|  | Block RAM Capacity (Mb) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XC7S6 | 180 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7S15 | 360 |  |  |  |  |  |  |  |  |  | Spartan-7 FPGAs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7S25 | 1620 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Speed grade |  |  |  |  |  |  |  |  |  | -1 |  | -2 |  |  |  |  |
| XC7S50 | 2700 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7S75 | 3240 |  |  |  |  |  |  |  |  |  | True dual-port Block RAM FMAX [MHz] |  |  |  |  |  |  |  |  |  | 388 | 461 |  |  |  |  |  |
| XC7S100 | 4320 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Artix-7 FPGAs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A12T | 720 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A15T | 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A25T | 1620 |  |  |  |  |  |  |  |  |  | Speed grade |  |  |  |  |  |  |  |  |  | -1 |  | -2 |  | -3 |  |  |
| XC7A35T | 1800 |  |  |  |  |  |  |  |  |  | True dual-port Block RAM FMAX [MHz] |  |  |  |  |  |  |  |  |  | 388 | 461 |  |  | 509 |  |  |
| XC7A50T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2700 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A75T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3780 |  |  |  |  |  |  |  |  |  | Kintex-7 and Virtex-7 FPGAs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A100T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A200T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 13140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Speed grade |  |  |  |  |  |  |  |  |  | -1 |  | -2 |  | -3 |  |  |
| XC7K70T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4860 |  |  |  |  |  |  |  |  |  | True dual-port Block RAM FMAX [MHz] |  |  |  |  |  |  |  |  |  | 458 | 544 |  |  | 601 |  |  |
| XC7K160T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 11700 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K325T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 16020 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K355T |  |  |  |  |  |  | 25740 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K410T |  |  |  |  |  |  | 28620 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K420T |  |  |  |  |  |  | 30060 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K480T |  |  |  |  |  |  | 34380 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7V585T |  |  |  |  |  |  | 28620 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7V2000T |  |  |  |  |  |  | 46512 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX330T |  |  |  |  |  |  | 27000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX415T |  |  |  |  |  |  | 31680 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX485T |  |  |  |  |  |  | 37080 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX550T |  |  |  |  |  |  | 42480 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX690T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 52920 |  |  |  |  |  |  |
| XC7VX980T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 54000 |  |  |  |  |  |
| XC7VX1140T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 67680 |  |  |
| XC7VH580T |  |  |  |  |  |  | 33840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VH870T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50760 |  |  |  |  |  |  |

For more information, refer to: UG473, 7 Series FPGAs Memory Resources User Guide

Important: Verify all data in this document with the device data sheets found at www.xilinx.com

Page 8

High-Speed Serial Transceivers

7 series devices provide a broad portfolio of transceivers for applications ranging from low-cost consumer products to high-end networking systems.

|  | Total Transceiver Count |  |  |  |  |  |  |  |  |  | GTP = 6.6 Gb/s |  |  |  |  |  | GTP |  |  | GTX |  |  | GTH |  |  | GTZ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XC7A12T |  |  |  |  |  |  |  |  |  |  | GTX = 12.5 Gb/s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | GTH = 13.1 Gb/s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A15T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A25T |  |  |  |  |  |  |  |  |  |  | GTZ = 28.05 Gb/s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A35T |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A50T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  |  |  | For more information, refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A75T |  | 8 |  |  |  |  |  |  |  |  | UG482, 7 Series FPGAs GTP Transceivers User Guide |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A100T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  |  |  |  |  |  | UG476, 7 Series FPGAs GTX/GTH Transceivers User Guide |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A200T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K70T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K160T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K325T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K355T |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K410T |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K420T |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K480T |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7V585T |  | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7V2000T |  | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX330T |  | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX415T |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX485T |  |  |  |  |  |  |  |  |  |  | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX550T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 |  |  |  |  |  |  |  |
| XC7VX690T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 |  |  |  |  |  |  |  |
| XC7VX980T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 72 |  |  |  |  |  |  |  |  |  |  |
| XC7VX1140T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 96 |  |  |
| XC7VH580T |  | 48 |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VH870T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 72 |  |  |  |  |  | 16 |  |  |  |  |

Important: Verify all data in this document with the device data sheets found at www.xilinx.com

Page 9

Transceiver Aggregate Bandwidth

7 series devices provide a broad portfolio of transceivers for applications ranging from low-cost consumer products to high-end networking systems.

|  | Transceiver Aggregate Bandwidth |  |  |  |  |  |  |  |  |  |  | GTP |  |  |  |  |  | GTX |  |  |  |  |  | GTH |  |  | GTZ |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XC7A12T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  |  |  |  |  |  |  | Maximum Line Rates |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A15T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A25T |  |  |  |  |  |  |  |  |  |  |  | GTP = 6.6 Gb/s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A35T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 52 |  |  |  |  |  |  |  |  |  | GTX = 12.5 Gb/s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A50T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 52 |  |  |  |  |  |  |  |  |  | GTH = 13.1 Gb/s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A75T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 105 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | GTZ = 28.05 Gb/s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A100T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 105 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A200T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 211 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | For more information, refer to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K70T |  |  |  |  |  |  |  |  |  |  |  | UG482, 7 Series FPGAs GTP Transceivers User Guide |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K160T |  |  |  |  |  |  |  |  |  |  |  | UG476, 7 Series FPGAs GTX/GTH Transceivers User Guide |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K325T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 400 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K355T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 600 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K410T |  | 400 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K420T |  |  |  |  |  |  | 800 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K480T |  |  |  |  |  |  | 800 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7V585T |  |  |  |  |  |  | 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7V2000T |  |  |  |  |  |  | 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX330T |  |  |  |  |  |  | 733.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX415T |  |  |  |  |  |  | 1257.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX485T |  |  |  |  |  |  | 1400 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX550T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2096 |  |  |  |  |  |
| XC7VX690T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2096 |  |  |  |  |  |
| XC7VX980T |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1886.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX1140T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2515.2 |  |  |  |
| XC7VH580T |  |  |  |  |  |  | 1257.6 |  |  |  |  |  |  |  | 448.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VH870T |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1886.4 |  |  |  |  |  |  |  |  |  |  | 896 |  |  |  |

Important: Verify all data in this document with the device data sheets found at www.xilinx.com

Page 10

I/O Count and Bandwidth

| XC7S6 |  |  | 125 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Total I/O Count |  |  |
| XC7S15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  | 125 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Total Bandwidth in Gb/s |  |  |
| XC7S25 |  |  |  | 187.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 150 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | I/O Bandwidth = Total I/O x LVDS Performance |  |  |
| XC7S50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 312.5 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7S75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 500 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 400 |  |  |  |  |  |  |  |  |  |  |  |
| XC7S100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 500 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 400 |  |  |  |  |  |  |  |  |  |  |  |
| XC7A12T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 187.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 150 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A15T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 312.5 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A25T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 187.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 150 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A35T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 312.5 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A50T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 312.5 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 250 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A75T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 375 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 300 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A100T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 375 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 250 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7A200T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 625 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 500 |  |  |  |  |  |  |  |  |
| XC7K70T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 480 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 300 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K160T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 640 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 400 |  |  |  |  |  |  |  |  |  |  |  |
| XC7K325T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 500 |  |  |  |  |  |  |  |  |
| XC7K355T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 480 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 300 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7K410T |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 500 |  |  |  |  |  |  |  |  |
| XC7K420T |  |  |  |  |  |  |  |  |  |  |  | 640 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 400 |  |  |  |  |  |  |  |  |  |  |  |
| XC7K480T |  |  |  |  |  |  |  |  |  |  |  | 650 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 400 |  |  |  |  |  |  |  |  |  |  |  |

| XC7V585T |  |  |  |  |  |  |  |  |  |  |  | 1360 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 850 |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7V2000T |  |  |  |  |  |  |  |  |  |  |  | 1680 |  |  |  |  |
|  |  |  |  |  |  |  | 1200 |  |  |  |  |  |  |  |  |  |
| XC7VX330T |  |  |  |  |  |  | 1120 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 700 |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX415T |  |  |  |  |  |  | 960 |  |  |  |  |  |  |  |  |  |
|  |  | 600 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX485T |  |  |  |  |  |  | 1120 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 700 |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX550T |  |  |  |  |  |  | 960 |  |  |  |  |  |  |  |  |  |
|  |  | 600 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX690T |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  |  |
|  |  |  |  |  |  |  | 1000 |  |  |  |  |  |  |  |  |  |
| XC7VX980T |  |  |  |  |  |  |  |  |  |  |  | 1440 |  |  |  |  |
|  |  |  |  | 900 |  |  |  |  |  |  |  |  |  |  |  |  |
| XC7VX1140T |  |  |  |  |  |  |  |  |  |  |  | 1760 |  |  |  |  |
|  |  |  |  |  |  |  | 1100 |  |  |  |  |  |  |  |  |  |
| XC7VH580T |  |  |  |  |  |  |  |  |  |  |  | Important: Verify all data in this document |  |  |  |  |
|  |  |  |  | 840 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 600 |  |  |  |  |  |  |  | with the device data sheets found at www.xilinx.com. |  |  |  |  |  |  |
| Page 11 XC7VH870T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 300 | 420 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

References

DS180, 7 Series FPGAs Overview

DS181, Artix-7 FPGAs Data Sheet: DC and AC Switching Characteristics

DS182, Kintex-7 FPGAs Data Sheet: DC and AC Switching Characteristics

DS183, Virtex-7 T and XT FPGAs Data Sheet: DC and AC Switching Characteristics

UG470, 7 Series FPGAs Configuration User Guide

UG471, 7 Series FPGAs SelectIO Resources User Guide

UG472, 7 Series FPGAs Clocking Resources User Guide

UG473, 7 Series FPGAs Memory Resources User Guide

UG474, 7 Series FPGAs Configurable Logic Block User Guide

UG475, 7 Series FPGAs Packaging and Pinout User Guide

UG476, 7 Series FPGAs GTX/GTH Transceivers User Guide

UG479, 7 Series FPGAs DSP48E1 Slice User Guide

UG480, 7 Series FPGAs and Zynq-7000 All Programmable SoC XADC Dual 12-Bit 1 MSPS ADC User Guide

UG482, 7 Series FPGAs GTP Transceivers User Guide

UG483, 7 Series FPGAs PCB Design Guide

| Important: Verify all data in this document with the device data sheets found at www.xilinx.com | XMP101 (v1.7) |
| --- | --- |

© Copyright 2014-2018 Xilinx, Inc. Xilinx, the Xilinx logo, Artix, ISE, Kintex, Spartan, Virtex, Vivado, Zynq, and other designated brands included herein are trademarks of Xilinx in the United States and other countries. PCI, PCIe, and PCI Express are trademarks of PCI-SIG and used under license. All other trademarks are the property of their respective owners.

Page 12